# HP Insight Management WBEM Management Processor Lite Provider

| Description | The HP Insight Management Web-Based Enterprise Management (WBEM) Management Processor Lite provider extends the management capability of referencing profiles by adding the capability to represent HP Management Processor on HP servers. This provider implements the following profiles and installs the necessary files: |  |
| --- | --- | --- |
| Profile Name | Organization | Version |
| HP Management Processor (MP) Lite Profile | HP WBEM TC | 2.0.1 |

| Requirements | For each hardware architecture listed, this provider requires the following distributions HP Integrity managed servers; SLES 10 and later RLEL 5.0 and later. HP ProLiant managed servers; SLES 11 and later RHEL 5.3 and later. |
| --- | --- |
| Release History | Initial release with HP Insight Management WBEM Providers for Linux v2.0. |

## Setting Up the Provider

| Installing the Provider | There are no special installation instructions for this provider. It is installed by default as part of the HP Insight Management WBEM providers. |
| --- | --- |
| Configuring the Provider | This provider does not accept specific configuration adjustments beyond standard HP Insight Management WBEM support. |

## Using the Provider

| Namespaces Supported by the Provider | This provider returns instances in the root/hpq namespace. |
| --- | --- |
| Schema Supported by the Provider | This provider supports the following classes: - SMX_ManagementProcessor - SMX_MPFirmware - SMX_MPCollection - SMX_MPHostedCollection - SMX_MPComponentCS - SMX_MPMemberOfCollection - SMX_MPElementFirmwareIdentity - SMX_MPInstalledFirmwareIdentity - SMX_MPConsolidatedStatusView - SMX_MPScopedConsolidatedStatusView |

The tables in the following sections describe the properties of the supported classes. The classes are categorized by the class or superclass that defines the property, the first column is the Property Name (including type and units) and the second column describes how the provider determines the properties implementation. When the Property Implementation value is a number, the number given is the default behavior and the Managed Object Format interpretation is within parenthesis. If other values are returned, a problem is indicated.

Unless otherwise noted, all of the property implementation values given are for HP ProLiant and HP Integrity (cellular and non-cellular) systems. The location related properties and implementation values are determined based on the server type so they may differ.

### SMX_ManagementProcessor Class

The SMX_ManagementProcessor class implements the HP_ManagementProcessor to describe a management processor controller system.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short Description of the Management Processor, including the product name and the enabled state of the MP. For example: Product name of the management processor in the format: <product name> (X) Where: <product name> is the name of the MP product and (X) indicates whether the MP is Active or Inactive/Disabled. For example: Integrated Lights Out 2 (iLO 2) (Active) Integrated Lights Out 2 (iLO 2) (Inactive) Integrated Lights Out 3 (iLO 3) (Active) |
| Description | Description of the Management Processor, including the product name. For example: HP Management Processor - <product name> Where: <product name> is the name of the MP product. For example: HP Management Processor – Integrated Lights Out 2 (iLO 2) |
| ElementName | Name of the Management Processor, including the product name and the enabled state of the MP. For example: Product name of the management processor in format: <product name> (X) Where: <product name> is the name of the MP product. (X)indicates whether the MP is Active or Inactive/Disabled For example: Integrated Lights Out 2 (iLO 2) (Inactive) Integrated Lights Out 2 (iLO 2) (Inactive) Integrated Lights Out 3 (iLO 3) (Active) |
| CIM_ManagedSystemElement |  |
| OperationalStatus | Enumerators indicating the Management Processor operational status (refer to OperationalStatus mapping table in HP Management Processor Lite Profile for specific descriptions of when each value is populated). OperationalStatus[0] contains MP operational status: 0 (Unknown), when NICCondition=0 2 (OK), normal operation 3 (Degraded), when NICCondition=5 6 (Error), when NICCondition=6 or MP software interface fails OperationalStatus[1] contains NIC status (when MP is enabled): 0 (Unknown), when NICCondition=0 or MP software interface fails 2 (OK), normal operation 3 (Degraded), when NICCondition=5 6 (Error), when NICCondition=6 10 (Stopped), when NICCondition=3 15 (Dormant), when NICCondition=4 Note: When a Management Processor is disabled (refer to property EnabledState) OperationalStatus[0] will always report a status of 2 (OK) so as not to contribute negatively to overall system status. An MP is disabled only if the user has manually disabled it via the MP’s RBSU settings; this is seen as a normal condition. In this scenario OperationalStaus[1] is not populated. |
| StatusDescriptions | Descriptive text per OperationalStatus. StatusDescriptions[0] text per OperationalStatus[0]: Management Processor Status: Unknown Management Processor Status: OK Management Processor Status: Degraded Management Processor Status: Error StatusDescriptions[1] text per OperationalStatus[1]: Management Processor NIC Status: Unknown Management Processor NIC Status: OK Management Processor NIC Status: NIC disconnected Management Processor NIC Status: NIC failed Management Processor NIC Status: NIC is disabled Management Processor NIC Status: Not in use, MP is utilizing host NIC |
| HealthState | 0 (Unknown), when OperationalStatus[0]=0 (Unknown) 5 (OK), normal operation 10 (Degraded/Warning), when OperationalStatus[0]=3 (Degraded) 20 (Major failure), when OperationalStatus[0]=6 (Error) Note: When a Management Processor is disabled (refer to property EnabledState) HealthState will always report a state of 5 (OK) so as not to contribute negatively to overall system status. An MP is disabled only if the user has manually disabled it via the MP’s RBSU settings; this is seen as a normal condition. |
| CIM_LogicalElement |  |
| CIM_EnabledLogicalElement |  |
| EnabledState | 2 (Enabled) for active MP 3 (Disabled) for inactive/disabled MP Note: The Active MP may be enabled or disabled (the user can manually disable the MP via RBSU settings). The Inactive MP is always disabled. |
| RequestedState | 12 (Not Applicable) |
| EnabledDefault | 2 (Enabled) |
| CIM_System |  |
| CreationClassName | SMX_ManagementProcessor |
| Name | Opaque instance identifier using HPQ as the OrgID. For example: HPQ:SMX_ManagementProccessor:1 |
| CIM_ComputerSystem |  |
| Dedicated | 14 (Management) |
| ResetCapability | 4 (Enabled) |
| HP_ManagementProcessor |  |
| ControllerType | Enumerator indicating the model or type of management processor. 0 (Unknown) 1 (Other) 2 (PCI Board-Remote Insight Lights Out Edition II (RILOE II)) 3 (Embedded Integrated Lights Out Controller (iLO)) 4 (Embedded Integrated Lights Out Controller 2 (iLO 2)) 5 (Embedded Integrity Management Processor) 6 (Embedded Integrity Integrated Lights Out Controller (iLO)) 7 (Embedded Integrity Integrated Lights Out Controller 2 (iLO 2)) 8(Embedded Lights Out 100 Series (LO 100)) 9(Embedded Integrated Lights Out Controller 3 (iLO 3)) 10(Embedded Integrity Integrated Lights Out Controller 3 (iLO 3)) |
| OtherControllerType | Descriptive text describing the controller type when ControllerType=1 (Other) |
| HardwareVersion | The hardware version of the management processor. |
| UniqueIdentifier | String which uniquely distinguishes the management processor controller from any other MP hardware. |
| IPAddress | The IP address of the MP-resident network interface. Note: When a Management Processor is disabled (refer to property EnabledState) IP Address will always be reported as 0.0.0.0. Network communication with the MP is not possible when the MP is disabled. |
| URL | The Uniform Resource Locator of the management processor controller’s web-based user interface. Note: When a Management Processor is disabled (refer to property EnabledState) URL will always be reported as https://0.0.0.0. Network communication with the MP is not possible when the MP is disabled. |
| SupportedProtocols | An array of values indicating the protocol(s) supported by the management processor controller. It is assumed that any protocol listed is active and can be utilized. |
| NICCondition | Enumerator indicating the condition of the MP-resident network interface connection. 0 (Unknown) 2 (OK) 3 (NIC disabled) 4 (NIC not in operation-alternate host NIC in use) 5 (NIC in operation but disconnected) 6 (Failed) Note: When a Management Processor is disabled (refer to property EnabledState) NICCondition will always report a state of 3 (NIC disabled ). Network communication with the MP is not possible when the MP is disabled. |
| ActiveLicense | Enumerator indicating the license type of the Management Processor. 0 (Unknown) 1 (None) 2 (iLO Advanced) 3 (iLO Light) 4 (iLO Advanced for Blade System) 5 (iLO standard for Blade System) Note: When communication failure with MP occurs then ActiveLicense is Unknown. If no license present then ActiveLicense is None. |
| LicenseKey | Management Processor License Key |
| HostName | The network hostname in use by this management processorif one is defined |
| IPv6Address | The IPv6 address of the management processorcontroller’s NIC if one is defined |
| IPv6SubnetPrefixLength | Describes the prefix length of the IPv6Address property in order to specify a valid IPv6 subnet |
| IPv4Address | The IPv4 address of the management processorcontroller’s NIC if one is defined |
| IPv4SubnetMask | The mask for the IPv4 address of the management processorcontroller’s NIC if an IPv4 address is defined |
| PermanentAddress | The permanent network address or MAC that is hard-coded for the management processor controller’s NIC |
| GatewayIPAddresses[] | The IPv4 and IPv6 addresses which define the default IPv4 and/orIPv6 gateways for the management processor |
| HostName | The network hostname in use by this management processo,rif one is defined |
| HP_ManagementProcessor |  |

### SMX_MPFirmware Class

The SMX_MPFirmware class implements the HP_MPFirmware class to describe management processor firmware

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Short Description of the MP firmware For example: Management Processor Firmware |
| Description | Description of the MP firmware. For example: HP Management Processor Firmware |
| ElementName | Represents the target type/firmware family. Examples: RI7 for iLO 2 R19 for iLO 3 |
| CIM_ManagedSystemElement |  |
| Name | Product name of the management processor. Examples: Integrated Lights Out 2 (iLO 2) |
| OperationalStatus | 2 (OK), normal operation 6 (Error), when MP software interface fails |
| StatusDescriptions | Management Processor Firmware Status: OK |
| HealthState | 5 (OK), normal operation 20 (Major failure), when MP software interface fails |
| CIM_LogicalElement |  |
| CIM_SoftwareIdentity |  |
| InstanceID | Opaque instance identifier using HPQ as the OrgID. For example: HPQ:SMX_MPFirmware:1 |
| MajorVersion | The major version of the MP firmware. |
| MinorVersion | The minor version number of the MP firmware. |
| VersionString | String indicating the MP firmware version information. This property is the string translation of the content provided in the MajorVersion and MinorVersion properties. |
| Manufacturer | Hewlett-Packard |
| Classifications | 10 (Firmware) |
| ClassificationDescriptions | HP Management Processor Firmware |
| IdentityInfoValue | Indexed array that provides the values described by the type information stored in the corresponding component of the IdentityInfoType array. For IdentityInfoType CIM:SoftwareFamily, the value will represent the target type/firmware family: Examples: HPQ:RI7 for ProLiant iLO 2 HPQ:RI9 for ProLiant iLO 3 HPQ:iLO2 for Integrity iLO 2 HPQ:MP for Integrity MP |
| IdentityInfoType | Indexed array that provides the Description of the type of information that is stored in the corresponding component of the IdentityInfoValue array. CIM:SoftwareFamily |
| ReleaseDate | The MP firmware date (in DMTF datetime format). |
| HP_MPFirmware |  |

### SMX_MPCollection Class

The SMX_MPCollection class implements the HP_MPCollection class to represent the collection of all management processor controller systems in a computer system and forwards group operational status.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Description | Description of the class, for example HP Management Processor Collection |
| ElementName | ElementName for the class For example: Management Processor Collection |
| CIM_Collection |  |
| CIM_SystemSpecificCollection |  |
| InstanceID | Opaque instance identifier using HPQ as the OrgID. For example: HPQ:SMX_MPCollection:1 |
| HP_GroupSystemSpecificCollection |  |
| Caption | Caption for the class, for example Management Processor Collection |
| GroupOperationalStatus | Describes the group operational status of the management processor collection (refer to HP Management Processor Lite Profile). The group operational status represents the “worst-of” algorithm for all individual MP statuses in HP_ManagementProcessor.OperationalStatus[0]. GroupOperationalStatus[0] contains group operational status: 0 (Unknown) 2 (OK) 3 (Degraded) 6 (Error) |
| GroupStatusDescriptions | GroupStatusDescriptions[0] text per GroupOperationalStatus[0]: Aggregate Management Processor Collection Status: Unknown Aggregate Management Processor Collection Status: OK Aggregate Management Processor Collection Status: Degraded Aggregate Management Processor Collection Status: Error |
| SMX_MPCollection |  |

### SMX_MPHostedCollection Class

The SMX_MPHostedCollection class implements the HP_MPHostedCollection class, which associates a management processor collection to the scoping computer system.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Dependency |  |
| CIM_HostedDependency |  |
| CIM_HostedCollection |  |
| HP_GroupHostedCollection |  |
| HP_MPHostedCollection |  |
| Antecedent | References HP_ComputerSystem |
| Dependent | References HP_MPCollection |

### SMX_MPComponentCS Class

The SMX_MPComponentCS class implements the HP_MPComponentCS class to associate a management processor subsystem to the scoping computer system that contains it.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Component |  |
| CIM_SystemComponent |  |
| CIM_ComponentCS |  |
| HP_MPComponentCS |  |
| GroupComponent | References HP_ComputerSystem |
| PartComponent | References HP_ManagementProcessor |

### SMX_MPMemberOfCollection Class

The SMX_MPMemberOfCollection class implements the HP_MPMemberOfCollection class to associate a management processor to the collection to which it belongs.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_MemberOfCollection |  |
| HP_MPMemberOfCollection |  |
| Included | Writeable boolean property indicating if the management processor instance referenced by Member will contribute status to the HP_MPCollection. - TRUE—Include member status in collection status - FALSE—Do not include member status in collection status |
| Collection | References HP_MPCollection |
| Member | References HP_ManagementProcessor |

### SMX_MPElementFirmwareIdentity Class

The SMX_MPElementFirmwareIdentity class implements the HP_MPElementFirmwareIdentity class to associate a management processor to its corresponding firmware.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Dependency |  |
| CIM_ElementSoftwareIdentity |  |
| UpgradeCondition | 3 (Owner Upgradeable) |
| ElementSoftwareStatus | ElementSoftwareStatus[0] 6 (Installed) ElementSoftwareStatus[1] 2 (Current) |
| HP_MPElementFirmwareIdentity |  |
| Antecedent | References HP_MPFirmware |
| Dependent | References HP_ManagementProcessor |

### SMX_MPInstalledFirmwareIdentity Class

The SMX_MPInstalledFirmwareIdentity class implements the CIM_MPInstalledFirmwareIdentity class to associate a management processor to its corresponding installed firmware.

The following table lists the properties implemented.

| Property Name | Property Implementation |
| --- | --- |
| CIM_InstalledSoftwareIdentity |  |
| SMX_MPInstalledFirmwareIdentity |  |
| System | References SMX_ManagementProcessor |
| InstalledSoftware | References SMX_MPFirmware |

### SMX_MPConsolidatedStatusView Class

HP_ConsolidatedStatusView extends the class CIM_View instances are used to aggregate and model all Consolidated Status information into a instances related to a single subsystem. If consolidated View Classes are implemented, there shall be one instance of HP_ConsolidatedStatusView for each instance of HP_GroupSystemSpecificCollection.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ManagedElement |  |
| Caption | Unique name that describes this consolidated status view class instance collection represents. |
| Generation | This uint64 rolling counter will wrap and uniquely identify a single instance’s change of state. |
| ElementName | Same value as Caption defined above |
| CIM_View |  |
| InstanceID | Opaque instance identifier HPQ as the OrgID Example: HPQ:SMX_MPConsolidatedStatusView:1 |
| HP_ConsolidatedStatusView |  |
| GroupCreationClassName | HP_GroupSystemSpecificCollection.CreationClassName value |
| GroupCreationOperationalStatus | current status value of the HP_GroupSystemSpecificCollection |
| MemberOperationalStatus[] | Status(es) of the members |
| MemberCreationClassNames[] | CIM_ManagedSystemElement.CreationClassName values |
| MemberCreationStatusIncluded[] | Boolean indicating whether the various CIM_ManagedSystemElement Included |
| MemberObjectPaths[] | Object path references to each CIM_ManagedSystemElement member of this collection |

### SMX_MPScopedConsolidatedStatusView Class

HP_ScopedConsolidatedStatusView extends the class CIM_ScopedView for use with HP_ComputerSystem and HP_ConsolidatedStatusView. There shall be one instance of HP_ScopedConsolidatedStatusView implemented for each HP_ConsolidatedStatusView instance.

.

| Property Name | Property Implementation |
| --- | --- |
| CIM_ScopedView |  |
| HP_ScopedConsolidatedStatusView |  |
| Antecedent | References HP_ComputerSystem |
| Dependent | References SMX_MPConsolidatedStatusView |

## Provider Indications

| Indications Generated by the Provider | The following tables describe the SMX WBEM MP Provider indications that are implemented for HP Consolidated Status View. |
| --- | --- |

### Provider Indication Common Properties

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| IndicationIdentifier | GUID string generated at the time of indication. |
| IndicationTime | Time of indication. |
| CIM_AlertIndication |  |
| EventTime | Time of the event or time of the indication if event time unknown. |
| SystemName | SMX_ComputerSystem.Name |
| SystemCreationClassName | SMX_ComputerSystem.CreationClassName |
| HP_AlertIndication |  |
| ProviderVersion | Provider Version in the format VV.UU.FF. For example: 01.05.00 |
| NetworkAddresses | Contains a list of all the IP addresses of the computer system generating the indication. |
| OSType | On ESX, 39 (VM) On Linux, 36 (Linux) |
| OSVersion | The operating system version of the computer system generating the indication in the following format: major.minor.build |
| SystemFirmwareVersion | Array of firmware versions of the computer system generating the indication. |
| SystemSerialNumber | Serial number of the computer system generating the indication. |
| SystemProductID | Product ID of the computer system generating the indication. |
| SystemModel | Model name of the computer system generating the indication. |
| SystemGUID | Platform GUID of the computer system generating the indication. |
| SystemVirtualUUID[] | If Synergy is enabled, this will contain the logical UUID for the system. |
| SystemVirtualSerialNumber[] | If Synergy is enabled, this will contain the logical Serial Number for the system. |
| EnclosureName | HP_BladeEnclosureCS.Name |
| RackName | Rack name if one exists |
| RackUUID | Rack Unique Identifier if one exists |
| BladeName | HP_ComputerSystem.Name |
| BladeBay | HP_BladeCSLocation.LocationInformation[0] |

### HP_AlertStatusIndication: Group System Specific Collection Policy Change

The following table describes Mandatory Group System Specific Collection Policy Changed indication. This indication alerts clients to underlying changes in managed element’s state propagation filtering policy and whether those elements participate actively in the OperationalStatus of an HP_GroupSystemSpecificCollection instance. This indication shall be sent in the following cases:

If a managed element’s state propagation filtering policy is changed by altering the association CIM_MemberOfCollection.Included Boolean property

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Informational) |
| CIM_AlertIndication |  |
| Description | HP_GroupSystemSpecificConsolidated operational status policy has been modified to monitor operational status from different set of managed elements. |
| AlertingManagedElement | WBEM Path of the HP_GroupSystemSpecificCollection instance |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 7 (Model Alert) |
| EventID | “1” |
| ProviderName | “HP MP” |
| RecommendedActions | “Inspect property arrays MemberObjectPaths and MemberStatusIncluded to discover new managed elements and whether each managed element status is included in collection operational status.” |
| HP_AlertIndication |  |
| Summary | “Consolidated View Class Collection operational status policy has changed “ |
| EventCategory | 1 (Other) |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | “This group status policy change is a caused by one of two changes: - Additional or removal of managed elements to an HP_GroupSystemSpecificCollection 2. Activation or deactivation of State Propagation Filtering for existing managed elements in HP_GroupSystemSpecificCollection.” |

### HP_AlertStatusIndication: Consolidated Status Changed-OK

The following table describes one of the conditional Consolidated Status Changed indications. These Consolidated Status Changed indications shall be implemented if not all managed elements associated to an HP_GroupSystemSpecificCollection support the device status changed indications. If all managed elements of the collection support the device status changed indications, then the Consolidated Status Changed indications should not be implemented. If HP_ConsolidatedStatusView is not implemented, then AlertingManagedElement shall represent the CIM Object Path of the HP_GroupSystemSpecificCollection instance.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 2 (Informational) |
| CIM_AlertIndication |  |
| Description | Consolidated View Class Collection operational status has transitioned to OK or participating members in the HP_GroupSystemSpeciticCollection have transitioned to OK. |
| AlertingManagedElement | CIM Object Path of the HP_ConsolidatedStatusView instance if implemented. Otherwise CIM Object Path of the HP_GroupSystemSpecificCollection. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 7 (Model Alert) |
| EventID | “2” |
| ProviderName | “HP MP” |
| RecommendedActions | None |
| HP_AlertIndication |  |
| Summary | “Consolidated View Class Collection operational status has transitioned to OK. “ |
| EventCategory | 1 (Other) |
| OtherEventCategory | “View Class Status Changed” |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | “Operational Status of participating members in this collection have transitioned to OK from Degraded or Error state. These Operational Status changes could have also affected the HP_GroupSystemSpecificCollection’s operational status. |

### HP_AlertStatusIndication: Consolidated Status Changed-Degraded

The following table describes one of the conditional Consolidated Status Changed indications. These Consolidated Status Changed indications shall be implemented if not all managed elements associated to an HP_GroupSystemSpecificCollection support the device status changed indications. If all managed elements of the collection support the device status changed indications, then the Consolidated Status Changed indications should not be implemented. If HP_ConsolidatedStatusView is not implemented, then AlertingManagedElement shall represent the CIM Object Path of the HP_GroupSystemSpecificCollection instance.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 3 (Degraded) |
| CIM_AlertIndication |  |
| Description | Consolidated View Class Collection operational status has transitioned to Degraded or participating members in the HP_GroupSystemSpeciticCollection are now Degraded or Failed. |
| AlertingManagedElement | CIM Object Path of the HP_ConsolidatedStatusView instance if implemented. Otherwise CIM Object Path of the HP_GroupSystemSpecificCollection.. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 7 (Model Alert) |
| EventID | “3” |
| ProviderName | “HP MP” |
| RecommendedActions | “Inspect this View Class MemberOperationalStatus, MemberStatusIncluded and MemberObjectPath arrays to indict degraded or failing collection members.” |
| HP_AlertIndication |  |
| Summary | “Consolidated View Class Collection operational status has transitioned to Degraded. “ |
| EventCategory | 1 (Other) |
| OtherEventCategory | “View Class Status Changed” |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | “Operational Status of participating members in this collection have transitioned to Degraded state. These Operational Status changes could have also affected the HP_GroupSystemSpecificCollection’s operational status.” |

### HP_AlertStatusIndication: Consolidated Status Changed-Error

The following table describes one of the conditional Consolidated Status Changed indications. These Consolidated Status Changed indications shall be implemented if not all managed elements associated to an HP_GroupSystemSpecificCollection support the device status changed indications. If all managed elements of the collection support the device status changed indications, then the Consolidated Status Changed indications should not be implemented. If HP_ConsolidatedStatusView is not implemented, then AlertingManagedElement shall represent the CIM Object Path of the HP_GroupSystemSpecificCollection instance.

| Property Name | Property Implementation |
| --- | --- |
| CIM_Indication |  |
| PerceivedSeverity | 5 (Major) |
| CIM_AlertIndication |  |
| Description | Consolidated View Class Collection operational status has transitioned to error. |
| AlertingManagedElement | CIM Object Path of the HP_ConsolidatedStatusView instance if implemented. Otherwise CIM Object Path of the HP_GroupSystemSpecificCollection. |
| AlertingElementFormat | 2 (CIMObjectPath) |
| AlertType | 7 (Model Alert) |
| EventID | “4” |
| ProviderName | “HP MP” |
| RecommendedActions | “Inspect this View Class MemberOperationalStatus, MemberStatusIncluded and MemberObjectPath arrays to indict failing collection member.” |
| HP_AlertIndication |  |
| Summary | “Consolidated View Class Collection operational status has transitioned to Error. “ |
| EventCategory | 1 (Other) |
| OtherEventCategory | “View Class Status Changed” |
| ProbableCause | 1 (Other) |
| ProbableCauseDescription | “Operational Status of participating members in this collection have transitioned to Error state and caused operational status for the for the collection to transition to Error.” |